CSC303/EEN210: Digital Circuits

Fall 2024-2025

Course Project: Design, Testing, and   Implementation of            a Light Control System

Phase 1: Truth Table for the Light Control System Logic

| Name | ID |
| --- | --- |
| Mengistu Animaw Assaye | 1093890 |

Date:10/10/2024

Instructor: Dr. Mohamed Moursi

Abstract

In a hospital room, there are four doors, each equipped with a light switch, and three white- and one yellow-coloured lights. The task is to design a digital logic circuit that allows any of the four light switches to control the state of the white lights. This means that regardless of which switch is used, it should be possible to turn all the white lights on or off. The state of the yellow light will be on if only two light switches are turned on. Additionally, the room features a display screen with three LEDs that indicate how many of the switches are in the” on” position. For example, if two switches are turned on, the LED display should show the ”010” state.

The first task of phase 1 is to determine the appropriate logic gates that will enable the specified behaviour. This includes controlling how the lights and display react to the switches by utilizing fundamental logic operations like AND, XOR, and OR. 
By making a truth table that lists every conceivable configuration of the switches along with the equivalent actions of the white lights, yellow light, and LED display, one may determine how the circuit operates. The intended result is shown in the truth table for every possible combination of the switch inputs. We first created the truth table by using strong theoretical background, so that we can test every potential combination and validate it by constructing it on a real circuit.

Since the outcomes of Phase 1 will serve as the basis for the consecutive Phase's actual physical execution of the circuit, Phase 1 might be considered the most crucial stage.

Based on this real-life situation we can show the truth table of four switches and LEDs as the following tables. “0” is switch off and “1” is switch on. The result is shown in the table 2 below.

When,

- No switch on, LED display ‘000’state

- One switch on, LED display ‘001’ state

- Two switches on, LED display ‘010’state

- Three switches on, LED display ‘011’state

- Four switches on, LED display ‘100’ state

LED -3 indicates MSB, LED-2 indicates middle bit and LED-1 indicates LSB.

Table 1: Truth table of light controlling systems of four switches combination and display of screen.

| Inputs |  |  |  | Out puts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Switch -1 | Switch-2 | Switch-3 | Switch-4 | White light | Yellow light | LED-3 | LED-2 | LED-1 | LED Display |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 000 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 001 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 001 |
| 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 010 |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 001 |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 010 |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 010 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 011 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 001 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 010 |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 010 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 011 |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 010 |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 011 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 011 |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 100 |